ESP32 HTTP Endpoint – Production Checklist (Extended Edition)

A practical validation and hardening guide for developers using the ESP32_IDF_HTTP_ENDPOINT Lite repository as a foundation for production-ready firmware.

Version: 1.0

# 1. Purpose of This Document

This checklist is designed to help embedded developers transition an ESP-IDF HTTP endpoint project from a functional demo (Lite) to a stable and production-ready implementation.

It focuses on validation, structure, reliability, security, and maintainability.

# 2. Network Initialization Validation

Wi-Fi credentials configurable via menuconfig or secure storage.

System waits for IP acquisition before starting HTTP server.

Reconnect logic implemented on Wi-Fi disconnection.

Proper event handler registration and cleanup.

IP address logged clearly at boot.

# 3. HTTP Server Initialization

httpd_start() return value checked.

Server configuration reviewed (stack size, task priority, max URI handlers).

Server starts only after network readiness.

Graceful shutdown logic available (httpd_stop).

Timeout values reviewed for expected use case.

# 4. Endpoint Architecture Review

Clear separation between HTTP layer and business logic.

Each URI handler performs minimal logic and delegates processing.

Query parameters validated before usage.

JSON responses structured consistently.

Error codes returned appropriately (400, 404, 500).

# 5. Memory and Resource Management

Heap usage monitored before and after HTTP requests.

No dynamic allocation without proper free().

Stack size validated under stress conditions.

Large buffers avoided or statically allocated when possible.

No memory leaks after repeated endpoint calls.

# 6. Error Handling and Input Validation

Missing parameters handled gracefully.

Invalid values return clear error messages.

Boundary conditions tested (max/min values).

Malformed HTTP requests tested.

Unexpected states logged with warning level.

# 7. Logging Strategy

Log levels configurable (INFO, WARN, ERROR).

Sensitive data not printed in logs.

HTTP request entry logged minimally.

Production build reduces verbose debug output.

# 8. Basic Security Hardening

Input sanitization applied to query strings.

Optional authentication mechanism considered (token/header-based).

Server bound only to required interface.

Rate limiting strategy evaluated.

HTTPS feasibility evaluated if required by product scope.

# 9. Functional Testing Scenarios

Test using curl or Postman:

curl "http://<ESP_IP>/api/led?state=on"

curl "http://<ESP_IP>/api/led?state=off"

curl "http://<ESP_IP>/api/led?level=1"

Correct JSON response returned.

GPIO/LED state matches request.

Multiple sequential requests do not crash system.

Invalid request does not reboot device.

# 10. Pre-Release Validation Checklist

Firmware built with production flags.

Watchdog enabled and tested.

Power-cycle tested multiple times.

Stress test performed (100+ requests).

Memory usage recorded and documented.

Version number incremented and documented.

# 11. Upgrade Path to Professional Template

If you are planning to scale this Lite HTTP example into a reusable product component, consider implementing:

Modular endpoint registration system.

Middleware layer abstraction.

Authentication and role management.

Structured JSON response framework.

Unit testable handler modules.

Integration with OTA system.

# 12. Final Notes

This extended checklist is intended to help developers avoid common pitfalls when exposing HTTP APIs on ESP32 devices. While the Lite repository provides a clean starting point, production firmware requires structured validation and disciplined architecture.

Use this document as a repeatable release validation template for all future HTTP-based projects.